Microbiologic Diagnosis of Tuberculosis

| Slide 1 |  | Instructor’s Guide Module: Microbiologic Diagnosis of Tuberculosis ISTC Standards covered: [Standards reviewed only in terms of microscopy, culture, and drug susceptibility testing in Standards 2-6, 10, and 14.] Module Time: 60 minutes Alternate slides: Template slides to list local diagnostic testing resources and contact information, Introductory ISTC slides Interactive options: Ideas for interactive discussions are offered on many of the slides in this module. Participant discussion can enhance active learning, but will add more time to the lecture and must be planned for. Additional material: Slides containing related material may be found in the following module: Clinical presentation and diagnosis of TB. Test Questions: May be attached or inserted within presentation for discussion purposes, or alternatively, combined with questions from other modules to produce evaluation tool. The full text of the ISTC and all supporting references are available at www.istcweb.org Other useful Resources/References: - Acid-fast direct smear microscopy training package, WHO/CDC/RIT/IUATLD/APHL, 2006. www.cdc.gov/dls/ila/acidfasttraining/default.aspx - Management of Tuberculosis Training for Health Facility Staff, WHO Stop TB Partnership 2003. www.who.int/docstore/gtb/publications/training/management_of_tb/index.htm - Pai M et al. New tools and emerging technologies for the diagnosis of tuberculosis: Part II. Active tuberculosis and drug resistance. Expert Rev. Mol. Diagn 2006; 6(3):423-432. Slide contribution: Ed Desmond, Ph.D., Microbial Diseases Lab, California Department of Health Services, Richmond, CA [Image Credits: Centers for Disease Control Public Health Image Library / Dr. George P. Kubica] |
| --- | --- | --- |
| Slide 2 |  | - [Review objectives from slide] |
| Slide 3 |  | - Overview: [Review content from slide] |
| Slide 4 |  | - Overview: [Review content from slide] [Image credit: Lung Health Image Library/Jan van der Hombergh] |
| Slide 5 |  | - Significance of microbiologic testing for public health goals and patient care: - An important barrier to global TB control is the low rate of case detection. - As part of the international WHO Stop TB Strategy, the overall target goal of 70% case detection of new smear-positive cases is dependent on appropriate access to and usage of microbiologic testing for M.tb. - The direct significance to the patient is: [Review content of slide] |
| Slide 6 |  | Important points to be noted regarding sputum smear microscopy: - For many areas of the world, sputum smear microscopy remains the most important and available test for the diagnosis of TB. - Direct staining of unconcentrated specimens for smears is the most common practice, but sensitivity can be enhanced with chemical processing and concentration techniques or with the use of fluorescence microscopy. - Quality assurance measures should be instituted and monitored routinely for all microscopy programs. [Transition: The following slides will review each of these important aspects.] [Image credit: Lung Health Image Library/Jad Davenport] |
| Slide 7 |  | - [Review content of slide] [Image credits: Lung Health Image Library/Jad Davenport] |
| Slide 8 |  | - [Review content of slide] |
| Slide 9 |  | - Oil-immersion view of a Ziehl-Neelsen stained sputum smear with M.tb bacilli staining red with inflammatory cells stained blue. (Note: concentrated specimen). - The photo shows the typical morphological features of M.tb. It is a slender, curved, beaded rod, often occurring in clumps. [Image Credit: CDC Public Health Image Library/Dr. George P. Kubica] |
| Slide 10 |  | - [Review content of slide] - Note that this level of magnification is lower that the level used with regular light microscopy, allowing the technician to review the entire smear in a more rapid, time-saving fashion. |
| Slide 11 |  | - Through a fluorescent microscope, the organisms will brightly fluoresce against a black background making them easy to identify (here with an Auromine-rhodamine stain). [Image credit: Microbial Diseases Laboratory, California Dept of Public Health/Ed Desmond, PhD] |
| Slide 12 |  | - Fluorescence microscopy is widely used in many parts of the world. - Advantages: A recent, comprehensive systematic review of 43 studies showed that fluorescence microscopy is more accurate. [Review content of slide] - The combination of increased sensitivity with little or no loss of specificity makes fluorescence microscopy a more accurate test, although the increased cost and complexity might make it less applicable in many areas. - For this reason, fluorescence staining is probably best used in centers with specifically trained and proficient microscopists, in which a large number of specimens are processed daily, and in which there is an appropriate quality control program. [Reference: Steingart KR et al. Fluorescence versus conventional sputum smear microscopy for tuberculosis: a systematic review. Lancet Infect. Dis. 2006; 6(9):570-81]. [Image credit: Lung Health Image Library/Pierre Virot] |
| Slide 13 |  | A variety of methods have been used to improve the performance of sputum smear microscopy. - In general, the sensitivity of microscopy (as compared to culture) is higher with concentration by centrifugation and/or sedimentation (usually after pretreatment with chemicals such as bleach, NaOH, and NaLC)* or both, as compared to direct (unconcentrated) smear microscopy. - A comprehensive, systematic review of 83 studies describing the effects of various physical and/or chemical methods for concentrating and processing sputum prior to microscopy found that concentration resulted in higher sensitivity (15-20%increase) and smear-positivity rate, when compared with direct smears. - Centrifugation is more complex, requires electrical power, and may be associated with increased infection risk to laboratory personnel. Consequently, it is not clear that the advantages offset the disadvantages in low-resource settings. [Interactive option: Ask participants if they know how their local facilities process their smears – direct vs. concentrated, chemical processing, and type of stain/microscopy available? Where are referral centers, what can be done there, when and how may specimens be sent?] *[Note: Bleach and sodium hydroxide (NaOH) are more toxic to oral flora than AFB and help to decontaminate the specimen. N-acetyl-L-cysteine (NALC) is a commonly used mucolytic agent that dissolves mucin and helps to liquefy respiratory specimens.] [Reference: Steingart KR, et al. Sputum processing methods to improve the sensitivity of smear microscopy for tuberculosis: a systematic review. Lancet Infect. Dis 2006; 6(10):664-74]. |
| Slide 14 |  | - [Review content of slide] - [It is important to adjust according to your country’s National Tuberculosis Program procedures for sputum collection.] |
| Slide 15 |  | - [Review content of slide] - [It is important to adjust according to your country’s National Tuberculosis Program procedures for sputum collection.] |
| Slide 16 |  | - The limitation of sputum smear microscopy is its sensitivity. - As illustrated in the table: compared with culture, sputum smear microscopy is 68% sensitive in detecting M. tuberculosis. - Of all specimens that are AFB positive nearly 86% are detected by examining one specimen and an additional 12% are found on the 2nd specimen; thus, the incremental yield of the 3rd specimen is very low. A similar increment is found for the sensitivity of the 2nd and 3rd specimens. - The yield is better with a single early morning specimen than with a spot specimen obtained at other times during the day. [Reference: Mase SR, et al. Yield of serial sputum specimen examinations in the diagnosis of pulmonary tuberculosis: a systematic review. Int J tuberc Lung Dis 2007;11(5): 485-95] |
| Slide 17 |  | - The central importance of obtaining sputum for microscopic examination is the diagnosis of TB is reflected in the following ISTC Standards of Diagnosis (2, 3, 4, and 10). - [Note: Guidelines have recently changed from three sputum smears to at least two sputum smears. The change is reflected above and differs from the wording in the original published ISTC.] - [Read Standard] |
| Slide 18 |  | - [Read Standard] |
| Slide 19 |  | - Repeat sputum microscopy plays an equally important role in the monitoring for response to treatment. - [Read Standard – Note only partial Standard 10, limited to issue of sputum microscopy] |
| Slide 20 |  | - Where resources permit and adequate, quality-assured laboratory facilities are available, culture should be included in the algorithm for evaluating patients with negative sputum smears. - Properly done, culture adds a significant layer of complexity and cost but also increases sensitivity, which should result in earlier case detection. - Although the results of culture may not be available until after a decision to begin treatment has to be made, treatment can be stopped subsequently if cultures from a reliable laboratory are negative, the patient has not responded clinically, and the clinician has sought other evidence in pursuing the differential diagnosis as above. [Image credit: Lung Health Image Library/Jad Davenport] |
| Slide 21 |  | Culture advantages: - Culture of specimens has a higher sensitivity than microscopy and, at least in theory, can increase case detection (although this potential has not been demonstrated in low-income, high-incidence areas). While sputum microscopy is likely to be positive when there are at least 10,000 organisms per ml of sputum, at concentrations below 1000 organisms/ml, the chance of a positive smear is less than 10%. - In contrast, a properly performed culture can detect far lower numbers of acid-fast bacilli with a detection limit of about 100 organisms per ml. - Culture should be included in the algorithm for evaluating patients with negative sputum smears because culture results may confirm a TB diagnosis. - Culture can also help to identify other mycobacterial species and allow for drug susceptibility testing (particularly important when drug-resistance is suspected). |
| Slide 22 |  | - [Review content of slide] - Because of the added complexity required for quality culture results, the test may only be available through referral. This should not discourage the use of culture when clinically indicated. - It is very important that proper TB treatment be started empirically in suspicious cases, rather than delay treatment while awaiting culture confirmation. Early empiric treatment reduces both morbidity and transmission of M.tb. |
| Slide 23 |  | - Culture methods using solid media is the reference standard. [Review content of slide] - [Note: Top image shows growth on Lowenstein Jensen slants and lower image is culture plate, also with Lowenstein Jensen media] [Image credits: San Francisco Public Health Laboratory] |
| Slide 24 |  | Liquid Media requires a much more sophisticated laboratory system, and where available, allows for a more rapid identification of culture positivity compared with solid media and may also be used for drug-susceptibility testing. - Liquid media, such as the BACTEC system utilize the release of radioactive CO2 from C-14 (carbon-14) labeled palmitic acid in the media to identify growth. - The MGIT system, also using liquid medium, has the advantage of having growth detected by the appearance of fluorescence in a silicone plug at the bottom of the tube, thereby avoiding radioactivity. - The use of either liquid technique can reduce time to culture results to 2-4 weeks, compared with 4-8 weeks for solid media. - Decisions to provide culture facilities for diagnosing TB and the methods to be used, clearly depend on financial resources, trained personnel, and the ready availability of reagents and equipment service. [Image credits: San Francisco Public Health Laboratory] |
| Slide 25 |  | - Accurate differentiation of M.tb from other non-tuberculosis mycobacterium can be done in the following ways: [Review content of slide] |
| Slide 26 |  | - Examples of identification by colony morphology: [Review content of slide] [Image Credits: CDC Public Health Image Library / Dr. George P. Kubica] |
| Slide 27 |  | - [Review content of slide] - Need to be aware that cross-contamination can occur and maintain suspicion for problems if the clinical scenario does not make sense and multiple positive results are found in specimens processed at the same time. - Direct communication with laboratory can be very helpful and is the only way to investigate this possibility. Quality control is essential to the functioning of mycobacteriology laboratories. - [Image – Example: use of same flask to add material to multiple specimens can lead to cross-contamination from one patient specimen to the next.] [Image credit: Microbial Diseases Laboratory, California Dept of Public Health/Ed Desmond, PhD] |
| Slide 28 |  | - As noted in Standard 5, obtaining cultures, if available, is particularly important in the diagnosis of suspected smear-negative TB. - [Read Standard] * Abbreviated on slide (Note: Because the fluoroquinolones are active against M. tuberculosis complex, and thus may cause transient improvement in persons with tuberculosis, they should be avoided) - [Note: Guidelines have recently changed from three sputum smears to at least two sputum smears. The change is reflected above and differs from the wording in the original published ISTC.] - It is important to remember that, on average, sputum smear microscopy is only approximately 50-70% sensitive when compared with culture. |
| Slide 29 |  | - Cultures also have an important role in the diagnosis of TB in children if smear-negative. - [Read Standard] |
| Slide 30 |  | - [Read Standard] (continued) |
| Slide 31 |  | - Once TB has been identified by culture growth, subcultures can be collected and tested for susceptibility to all of the first-line, and many of the second-line anti-TB drugs. Both solid and liquid culturing techniques may be used for drug-susceptibility testing (DST). - The basic principle used in the test methods is that culture growth is attempted in the face of antibiotic challenge. If the organism is susceptible, it will not grow in the presence of the antibiotic. - Example above shows agar plates divided into four segments. Each white disc has a different first-line drug impregnated onto it. Areas with no growth indicate drug susceptibility, areas with active growth indicate drug resistance. - DST to the first-line drugs should be performed in specialized reference laboratories that participate in an ongoing, rigorous quality assurance program. DST for first-line drugs is currently recommended for all patients with a history pf previous anti-TB treatment: patients who have failed treatment, especially those who have failed a standardized re-treatment regimen, and chronic cases are the highest priority. [Image credit: Microbial Diseases Laboratory, California Dept of Public Health/Ed Desmond, PhD] |
| Slide 32 |  | - Important indicators for the use of drug-susceptibility testing are outlined in ISTC Standard 14. - [Read Standard] |
| Slide 33 |  | - Important indicators for the use of drug-susceptibility testing are outlined in ISTC Standard 14. - [Read Standard] (continued) |
| Slide 34 |  | Tests to identify TB more rapidly then standard biochemical assays are available, but because of the high cost and technical requirements, are neither feasible nor practical in many settings. - The non-amplified DNA Probe tests are one example, and offer rapid identification of type of mycobacteria/species once growth is detected on culture. - NAAT testing is used primarily on smear-positive sputum. These tests can be used directly on clinical specimens to verify the presence of M.tb with results in <1 day. They have a high specificity and positive-predictive value for pulmonary disease, therefore allowing confidence in ruling-in disease, but sensitivity is lower and highly variable. - In countries with high-endemic rates of TB, smear-positive results already confer a high likelihood of TB, and the added value of NAAT testing is questionable. The utility of NAAT testing may be more appropriate for low-burden, high-resource settings. |
| Slide 35 |  | It is clear that there is a great need for more rapid and accessible testing for TB. Other rapid or more advanced tests currently in use or still under investigation include: - LAMP tests still being evaluated, but may be more promising for low-resource settings (do not require thermal cycler for PCR amplification). - Immunologic tests are more readily available via commercial kits, but are not yet accurate enough to replace microscopy and culture. |
| Slide 36 |  | Another example of rapid diagnostic tests: - HPLC is a highly technical and costly technique, and available in a limited setting even in high-resource areas. |
| Slide 37 |  | With the rising rates of MDR and XDR, the need for rapid and accurate identification of drug resistance is significant. Promising tests currently in use or under investigation include: - Line probe assays: DNA from M.tb directly from sputum samples can be amplified and identified. Probes can both identify M.tb and test for drug resistance based on the presence of genetic mutations associated with specific drug resistance (isoniazid and rifampicin). Has a high sensitivity (>=97%) and specificity (>=99%) for rifampicin resistance. Overall accuracy for detection of MDR equally high at 99%, and retained when rifampicin resistance alone was used as a marker for MDR. - Image shows examples of GenoType MTDBRplus strips (Hain Lifescience, Nehren, Germany). [Resource: WHO Policy Statement, Molecular line probe assays for rapid screening of patients at risk of multi-drug resistant tuberculosis (MDR-TB), 2008. www.who.int ] - Other rapid drug susceptibility tests – examples listed here are in use or under investigation in a more limited setting, most requiring more technical laboratory capabilities. [Image credit: Rapid Molecular Screening for Multidrug-Resistant Tuberculosis in a High-Volume, Barnard et al. Am. J. Respir. Crit. Care Med 2008; 177: 787-792] |
| Slide 38 |  | - In summary, important points regarding the microbiologic diagnosis of TB discussed: [Review content of slide] [Continued] [Image credits: Lung Health Image Library/Jad Davenport (top); Microbial Diseases Laboratory, California Dept of Public Health/Ed Desmond, PhD] |
| Slide 39 |  | - In summary, important points regarding the microbiologic diagnosis of TB discussed: [Review content of slide] [End] [Image credits: Lung Health Image Library/Jad Davenport (top); Microbial Diseases Laboratory, California Dept of Public Health/Ed Desmond, PhD] |
| Slide 40 |  | - Alternate Slides: Offer additional options that may be added or substituted into module. - See Facilitator’s Guide for further information. |
| Slide 41 |  | - [Optional slide: Use this slide to add specific information on where and what type of testing (those listed above or others) is available to the course participants.] - [Interactive option – ask participants what resources have they utilized, share views on how to optimize the proper use of the diagnostic tests available.] |
| Slide 42 |  | - [Optional slide: Use this slide to add in specific information who and how to contact local/regional/reference laboratory resources for questions]. |
| Slide 43 |  | The International Standards for Tuberculosis Care (ISTC): - Are intended to unite public and private sectors in providing a uniformly accepted level of care for all patients with, or suspected of having, TB. - Describes the essential elements of TB care that should be available everywhere. - Provides a vehicle for mobilizing professional societies globally in support of TB programs. - Serves as a powerful advocacy tool to ensure that the essential elements are available. [Image Credit: Lung Health Image Library/Gary Hampton] |
| Slide 44 |  | - The ISTC consist of 17 evidence-based standards. - Standards differ from existing guidelines in that standards present what should be done, whereas, guidelines describe how the action is to be accomplished. - To meet the requirements of the Standards, approaches and strategies, determined by local circumstances and practices and developed in collaboration with local and national public health authorities, will be necessary. There are many situations in which the level of care can, and should, go beyond what is specified in the Standards. - The Standards should be viewed as a living document that will be revised as technology, resources, and circumstances change. Revisions to the original document released December 2005 are currently underway. - Funded (Oct 1, 2004) by USAID via TBCTA, development was supervised by an international steering committee (28 members / 14 countries) chosen to represent perspectives relevant to tuberculosis care and control. - The Standards are also intended to serve as a companion to and support for the Patients’ Charter for Tuberculosis Care developed in tandem with the Standards. - A Handbook for using the International Standards for Tuberculosis Care is also available (2007). The Handbook presents suggestions and guidance, based mainly on country-level experiences, for using the ISTC as a tool to foster and guide the delivery of high-quality care by all practitioners providing TB care. [Resource: www.istcweb.org] |
| Slide 45 |  | - The Standards are addressed to all healthcare providers, private and public, who care for persons with proven tuberculosis or with symptoms and signs suggestive of TB. - Three categories of activities are addressed by the Standards: diagnosis, treatment, and public health responsibilities of all providers. The ISTC are intended to be complementary to local and national TB control policies that are consistent with the World Health Organization (WHO) recommendations. - In many parts of the world there is great variability in the quality of tuberculosis care, and poor quality care continues to plague global tuberculosis control efforts. Effective engagement of all providers in providing high quality care in collaboration with TB control programs is key to the promotion of sound tuberculosis control. |
| Slide 46 |  | - Questions: May be used for interactive discussion, course evaluation, or continuing medical education purposes. - See Facilitator Guide for further information. |
| Slide 47 |  | Correct Answer: B |
| Slide 48 |  | Correct Answer: B |
| Slide 49 |  | Correct Answer: D |